VIDMODE.EXE, version 1.0

VIDMODE is a small utility for examining and changing the video mode of

EGA, VGA and MCGA graphics cards. The usage is

vidmode [options]

where the options are the following:

* No options   - Print the current video mode and screen size to standard

output.

* ?, h, help   - Show usage.

* <number>     - Switch to video mode <number> (in hex notation).

* #<number>    - Switch to video mode <number> (in decimal notation).

* mda, hgc     - Switch to MDA/Hercules mono 25 line mode (video mode 7).

* ega, vga     - Switch to CGA/EGA/VGA/MCGA 25 line mode (video mode 2/3).

* ega <number> - EGA 25/35/43 line text mode.

* vga <number> - VGA 12/14/16/20/21/25/28/29/30/33/34/35/40/43/48/50/60 line mode.

* e<number>    - Same as "ega <number>".

* v<number>    - Same as "vga <number>".

* AltPrtScr    - Install alternate (video BIOS) PrtSc routine.

* Alt          - Same as above.

Additionally, VIDMODE returns the current video mode in the ERRORLEVEL, and

so may be useful inside batch files.

VIDMODE is special in that it supports more than the usual number of VGA text

modes, all of which should work on any standard VGA card. The text modes can

be classified using two criteria, vertical resolution (# of scan lines) and

font:

Resolution:

200 scan lines (CGA) - 12x80, 14x80, 16x80, 20x80.

350 scan lines (EGA) - 21x80, 25x80 (in EGA cards), 29x80, 35x80, 43x80.

400 scan lines (VGA) - 25x80, 28x80, 33x80, 40x80, 50x80.

480 scan lines (VGA) - 30x80, 34x80, 48x80, 60x80.

Font:

ROM  8x16 - 12x80, 21x80, 25x80 (in VGA cards), 30x80.

ROM  8x14 - 14x80, 25x80 (in EGA cards), 28x80, 34x80.

User 8x12 - 16x80, 29x80, 33x80

User 8x10 - 20x80, 35x80, 40x80, 48x80.

ROM  8x8  - 20x80, 43x80, 50x80, 60x80.

The user-defined 8x10 font is generated from the ROM 8x8 font by adding a

vertical space above and below each character, except for line drawing

characters which are continued. This makes the font quite useful at the

high line number modes (40x80, 48x80), since the usual 8x8 font is very

crowded and can be hard to read. Similarly, the 8x12 font is generated

from the 8x14 ROM font. The combinations of 200 scan lines with

a 8x8 font (CGA 25x80) and 350 scan lines with a 8x14 font (EGA 25x80)

is not implemented, as it is not useful on a VGA adapter. The combination

of 8x12 font with 480 scan lines results in a 40x80 mode, which is already

implemented using the 8x10 font and 400 scan lines, and so does not appear.

I hope you will find VIDMODE useful. However, you use it entirely at your

own risk.

Enjoy!

Eyal Doron, doron@mickey.mpi-hd.mpg.de